# PROCEDURAL REQUIREMENTS

# Project Owner

The contract for special inspections services shall be directly between the owner and the special inspections agency. Neither the contractor nor any of their subcontractors shall have any monetary involvement in special inspections.

The owner shall ensure that copies of the County reviewed plans, specifications and shop drawing are provided to the special inspector prior to the start of the affected work.

The owner shall immediately notify the County Building Department if there is a change in special inspections agencies or a disruption in special inspections services.

# General Contractor

The contractor is responsible for notifying the special inspections agency when the work is ready for inspection.  A minimum of 24-hours’ notice shall be provided so that the special inspector has time to inspect the work prior to concealment.  The contractor shall provide access to and means for safe and proper inspection of the work.

The general contractor shall create a file (3 ring binder) for the special inspector daily reports.  This file shall be located in a conspicuous place in the project trailer/office to allow review by Building Department staff and remain in place until all work requiring special inspections in completed.

# Special Inspections Agency

It is the special inspector’s responsibility to review the county reviewed plans thoroughly and sufficiently in advance of construction to establish that adequate information is available to conduct the required inspections and tests.  All errors and/or omissions in the approved plans that create any form of uncertainty or ambiguity shall be resolved through the architect or engineer of record.

A Building Department approved special inspector shall perform inspections and/or tests of the work for conformance with the county reviewed plans, specifications, shop drawings and applicable provisions of the International Building Code.  It is the special inspector’s responsibility to verify that all work requiring special inspections is inspected and/or tested prior to concealment.

After each inspection, the special inspector shall furnish the contractor with a completed Daily Report Form.  The form shall indicate the type and location of the inspections/test performed and is signed by an “approved” special inspector.  Any non-conforming items shall be brought to the immediate attention of the general contractor and noted on the Daily Report Form.

The special inspector of record shall submit a report to the engineer and/or architect of record biweekly until all work requiring special inspections is complete.  This report shall be submitted on the Building Department Special Inspections Biweekly Report form and include:

A brief summary of the work performed during the reporting time frame.

Changes and/or discrepancies with the reviewed drawings or specifications that were observed during the reporting period.

Discrepancies which were resolved or corrected.

A list of nonconforming items requiring resolution. All applicable test results.

When the work requiring special inspections is completed and all nonconforming items have been resolved, the special inspector of record shall submit the Final Special Inspections Report to the Building Department and engineer and/or architect of record. A Certificate of Occupancy will not be issued until the final report has been reviewed and approved by the County Building Department.

Clear Creek County

Special Inspections Letter of Intent

has been retained by                                                                         to perform

SI Company Name	Owner or Owner's Authorized Agent

special inspections and tests as required on                          of the plans for construction per the

Sheet Number

applicable schedule and standard.                                          attests that personnel which will perform

SI Company Name

inspections are adequately experienced, educated, or otherwise qualified to perform the special inspections referenced. As an “Approved” special inspection agency we shall remain competent, objective, and independent from the contractor responsible for the work being inspected.  All work inspected shall be in conformance with the Clear Creek County Building Department reviewed plans, any deviation from Building Department reviewed plans shall be reviewed by the Building Department for approval. We have reviewed the Procedural Requirements and our agency, inspectors, and employees can identify the Clear Creek County Building Department watermark, if the construction documents do not contain the watermark they shall not be used for inspection.

Signed:  ________________________________________Date: _________________

Seal

SPECIAL INSPECTOR DAILY REPORT

Project Name:____________________________________________Date: ___________________

Project Address: ________________________________________Permit #: ________________

Indicate the items inspected and/or tested:

REINFORCED CONCRETE

[ ] Placement of Concrete

[ ] Placement of Shotcrete

[ ] Testing of Concrete

[ ] Reinforcing Steel Placement

[ ] Bolts Installed in Concrete

[ ] Prestressing Concrete

SOILS AND FOUNDATIONS

[ ] Verification of Soils

[ ] Excavation

[ ] Structural Fill

[ ] Piles and/or Piers

[ ] Detention Basin

[ ] Earth Retaining Structure

STRUCTURAL MASONRY

[ ] Inspection of Rebar Placement/Grouting

[ ] Mortar and Grout Testing

[ ] Wall Prisms

SPECIAL:

[ ] Post-Installed Anchors    [ ] Smoke Control

STRUCTURAL STEEL

[ ] High Strength Bolting

[ ] Welding of Structural Steel

[ ] Metal Deck Welding

[ ] Shear Stud Welding

[ ] Welding of Reinforcing Steel

[ ] Steel Frame Inspection

SPRAYED FIRE-RESISTANT MATERIALS

OR MASTIC AND INTUMESCENT FIRE

RESISTIVE COATINGS

[ ] Placement Inspection  [ ] Bond Strength

[ ] Density Tests    [ ] Substrate Temperature

[ ] Thickness Tests

PRECAST CONCRETE

[ ] Inspection of Erected Panels

[ ] Welding of Panel Connections

INSPECTION OF FABRICATORS

[ ] Metal Building  [ ] Structural Steel

[ ] Wood Const.     [ ] Precast Concrete

[ ] Firestop                              [ ]EIFS

[ ] Seismic Resistance           [ ] Wood Construction

[ ] OTHER: ______________________________________________________________________

List locations of inspections/test made:_________________________________________________ ________________________________________________________________________________

1.  Were there any discrepancies with the reviewed plans? 	 [ ] Yes  [ ] No

2.  Were there any changes to the reviewed plans? 	                [ ] Yes  [ ] No

- Were any previously listed items corrected or resolved  	 [ ] Yes  [ ] No

- Are the signed/sealed resolutions by EOR/AOR attached  	 [ ] Yes  [ ] No

If yes, on items 1, 2, or 3 describe:

_________________________________________________________________________________

_________________________________________________________________________________

___________________________________________

Special Inspections Agency:  _________________________________________________________

Inspector: _________________________  Signature: ______________________________________

(Print)

Time Beginning Inspection: ___________ Time Ending Inspection: ___________________________

BIWEEKLY SPECIAL INSPECTIONS REPORT

Project Address: _________________________________Permit: ___________________ Project Name:______________________________________________________________

Company Name:____________________________________________________________

This report covers work done between __________________ and ____________________

This is to certify that I, or a qualified individual working under my direction, inspected and/or tested the following items in accordance with Chapter 17 0f the 2018 International Building Code: (Check appropriate

items)

[ ] Placement of Reinforced Concrete

[ ] Placement of Shotcrete

[ ] Testing of Reinforced Concrete

[ ] Placement of Reinforcing Steel

[ ] Placement of Prestressing Steel

[ ] Bolts Installed in Concrete

[ ] Post-Installed Anchors

[ ] Verification of Soils

[ ] Excavation and Filling

[ ] Deep Foundations

[ ] Earth-Retaining Structure

[ ] Detention Basin

[ ] EIFS

[ ] Other

[ ] Smoke Control

[ ] Shotcrete

[ ] Erection of Precast Concrete

[ ] Steel Frame Inspection

[ ] High Strength Bolting

[ ] Structural Welding

[ ] Sprayed Fire-Resistant Materials or

Mastic and Intumescent Fire-Resistive

Coating

[ ] Structural Masonry

[ ] Wood Construction

[ ] Seismic Resistance

[ ] Firestop

INSPECTION OF FABRICATORS

[ ] Metal Building  [ ] Structural Steel

[ ] Wood Const.     [ ] Precast Concrete

Except where noted in the attached report, the work was found to be in substantial compliance with the County reviewed plans, specifications, and applicable provisions of the Clear Creek County Building Code.  All daily reports and EOR/AOR resolutions are attached.

Signed: ______________________________________________Date: _________________

Seal

FINAL SPECIAL INSPECTIONS FORM

Project Address:_________________________________Permit ___________________

Project Name:______________________________________________________________

Company Name:____________________________________________________________

Special Inspector of Record:___________________________________________________

This is to certify that I or qualified individuals working under my direction inspected and/or tested the following items in accordance with Section 1705 of the 2021 International Building Code:

| [ ] Placement of Reinforced Concrete [ ] Placement of Shotcrete [ ] Testing of Reinforced Concrete [ ] Placement of Reinforcing Steel [ ] Prestressing Concrete [ ] Post-Tension Slab [ ] Bolts installed in Concrete [ ] Post-Installed Anchors [ ] Verification of Soils [ ] Excavation and Filling [ ] Deep Foundations [ ] Earth Retaining Structure [ ] Detention Basin | [ ] EIFS Insulation/Finish System [ ] Smoke Control [ ] Erection of Precast Concrete [ ] Steel Frame Inspection [ ] High Strength Bolting [ ] Structural Welding [ ] Sprayed Fire Resistant Materials or Mastic and Intumescent Fire Resistive Coating [ ] Structural Masonry [ ] Wood Construction [ ] Seismic Resistance [ ] Firestop INSPECTION OF FABRICATORS [ ] Metal Building [ ] Structural Steel [ ] Wood Const. [ ] Precast Concrete |
| --- | --- |

[ ] Other: __________________________________________________________________

The entire scope of work for the items checked above is complete and was inspected by this

Agency, and to the best of my knowledge was found to be in substantial compliance with the County reviewed plans and specifications and applicable provisions of the Clear Creek County Building Code.

Signed:  ________________________________________Date: _________________

Seal